HARD CODED PROMPTS ANSWERED FOR REPORT INPUTS

OBSERVATIONS PROMPTS

Q1: How much is the rise in current envelope’s absolute value compared to best of its performance.

Ans: Probable answer expected from agent: Overall envelope value has increased to X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%.

Q2: How much is the rise in current acceleration’s absolute value compared to best of its performance.

Ans: Probable answer expected from agent: Overall Acceleration value has increased to X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%.

Q3: How much is the rise in current Velocity’s absolute value compared to best of its performance.

Ans: Probable answer expected from agent: Overall Velocity value has increased to X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%.

Q4: How much is the rise in current Total harmonic energy’s absolute value compared to best of its performance.

Ans: Probable answer expected from agent: Total harmonic energy value has increased to X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%.

Q5: Which Fault frequency’s total harmonic energy has increased highest in the current values?

Ans: Probable answer expected from agent: Total harmonic energy value has increased to for either of BPFI, BPFO, BSF or FTF from X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%. Other has also increased as follow; all fault frequencies increase in %.

Q6: How much is the rise in current Total Side band energy’s absolute value compared to best of its performance.

Ans: Probable answer expected from agent: Total sideband energy value around BPFI has increased to X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%. Other has also increased as follow; all fault frequencies increase in %.

Q7: How much is the rise in Kurtosis in the current acceleration time waveform compared to best of its performance?

Ans: Probable answer expected from agent: Kurtosis value has increased in the current time waveform from X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%.

Q8: How much is the rise in Crest factor in the current acceleration time waveform compared to best of its performance?

Ans: Probable answer expected from agent: Crest factor value has increased in the current time waveform from X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%.

ANALYSIS PROMPTS: analysis has to be drawn from the observations only

Q1: How much is the rise in current envelope’s absolute value compared to best of its performance.

Ans: Probable answer expected from agent: Overall envelope value has increased to X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%.

ANALYSIS: This point is only to show how much is rise in absolute values, therefore no analysis is required on this point.

Q2: How much is the rise in current acceleration’s absolute value compared to best of its performance.

Ans: Probable answer expected from agent: Overall Acceleration value has increased to X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%.

ANALYSIS: This point is only to show how much is rise in absolute values, therefore no analysis is required on this point.

Q3: How much is the rise in current Velocity’s absolute value compared to best of its performance.

Ans: Probable answer expected from agent: Overall Velocity value has increased to X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%.

ANALYSIS: This point is only to show how much is rise in absolute values, therefore no analysis is required on this point.

Q4: How much is the rise in current Total harmonic energy’s absolute value compared to best of its performance.

Ans: Probable answer expected from agent: Total harmonic energy value has increased to X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%.

ANALYSIS: Total harmonic energy rise indicate increased harmonical activities and represent fault in the bearing components.

Q5: Which Fault frequency’s total harmonic energy has increased highest in the current values?

Ans: Probable answer expected from agent: Total harmonic energy value has increased to for either of BPFI, BPFO, BSF or FTF from X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%. Other has also increased as follow; all fault frequencies increase in %.

ANALYSIS: Total harmonic energy rise is majorly increased due to rise in BPFO (it could be any based on highest contributor in total harmonic energy) indicate some fault in the bearing components (whichever is highest).

Q6: How much is the rise in current Total Side band energy’s absolute value compared to best of its performance.

Ans: Probable answer expected from agent: Total sideband energy value around BPFI has increased to X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%. Other has also increased as follow; all fault frequencies increase in %.

ANALYSIS: Total sideband energy rise indicate increase in modulation of running speed around the central frequency of BPFI. This could be due to damage in the race ways.

Q7: How much is the rise in Kurtosis in the current acceleration time waveform compared to best of its performance?

Ans: Probable answer expected from agent: Kurtosis value has increased in the current time waveform from X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%.

ANALYSIS: increasing Kurtosis value is an indication of irregularities in the race ways and subsequent impacting in the signals. This could be due to damage in the race ways.

Q8: How much is the rise in Crest factor in the current acceleration time waveform compared to best of its performance?

Ans: Probable answer expected from agent: Crest factor value has increased in the current time waveform from X value (current value) compared to Y value (which is best or the lowest value). This rise is around Z%.

ANALYSIS: increasing crest factor value is an indication of irregularities in the race ways and signal peakiness. This could be due to damage in the race ways.

"	"look for the Acceleration overall value and compare with the good value for % increase. 50%

"	Compare the overall value between current value and good value 0%

"if harmonics are increasing from the lowest spectrum harmonics by 10-20% in

•  Outer race: strong BPFO (10-20% from good data) and harmonics; fewer sidebands; peaks stable

•  Inner race: strong BPFI (10-20% from good data) with distinct sidebands at Running rpm (modulation).

•  Ball/roller (BSF): BSF and harmonics (10-20% from good data) , sometimes 2×BSF stronger; sidebands at running rpm.

•  Cage (FTF): prominent FTF (10-20% from good data)  and harmonics; often at lower frequency region.

Any harmonic is satisfying the above criterion shall be treated for abnormality; means either of or all of the harmonics up to 5 harmonics"	TWF impacting is slightly increasing and Kurtosis is 20% higher than good data kurtosis

Check for, fault frequency (as central frequency) sidebands of running RPM, its growth compared with good values spectrum. Use Side band energy tracking for the same. Energy increased by 10-20% from good data is making bearing health deteriorated compared to good bearing	Check for the crest factor comparison between current and good data and if it is showing increasing trend along with kurtosis rise of 20% consider impacting